## Multi-Camera Person Re-Identification & Tracking

### “One Identity.   Multiple Views.   Zero Confusion”

# 1. Problem Statement

Develop a real-time system that can detect, track, and follow a person across multiple camera feeds. The system should allow an operator to select a person and maintain tracking continuity while enabling PTZ cameras to automatically follow the selected individual.

# 2. Objective

The objective is to build a lightweight, real-time multi-camera tracking system that:

- Detects people in live RTSP streams

- Tracks multiple individuals per camera

- Maintains identity within each camera stream

- Allows manual selection of a target person

- Automatically controls PTZ cameras to follow the selected target

# 3. System Overview

The system processes multiple camera feeds simultaneously and performs:

- Person Detection using a deep learning model

- Multi-Object Tracking within each camera

- User-Based Target Selection via mouse input

- PTZ Camera Control for automated tracking

# 4. Key Features

## 4.1 Video Ingestion

- Supports multiple RTSP camera streams

- Each camera is processed in parallel using threaded frame readers

- Ensures low-latency frame access

## 4.2 Person Detection

- Uses YOLOv8 model for detecting humans

- Filters detections based on confidence threshold

- Outputs bounding boxes for each detected person

## 4.3 Multi-Person Tracking

- Uses DeepSORT for tracking individuals

- Assigns unique track IDs per person

- Maintains identity across frames within a camera

## 4.4 Target Selection

- Operator can click on a person in the video window

- Selected person becomes the active tracking target

- System highlights the selected individual

## 4.5 PTZ Camera Control

- Supports ONVIF-compatible PTZ cameras

- Automatically adjusts camera pan/tilt to keep target centered

- Uses smoothing and dead-zone logic to:

  - Reduce jitter

  - Prevent unnecessary movement

  - Ensure stable tracking

## 4.6 Real-Time Visualization

- Displays camera feeds using OpenCV windows

- Shows bounding boxes and IDs

- Highlights selected target

# 5. System Architecture (Simplified)

RTSP Streams

↓

Frame Reader (Threaded)

↓

YOLO Detection

↓

DeepSORT Tracking

↓

Target Selection (User Click)

↓

PTZ Control Logic

↓

Display Output (OpenCV)



# 6. Technology Stack

- Python

- OpenCV (video processing & display)

- YOLOv8 (Ultralytics) (person detection)

- DeepSORT (multi-object tracking)

- ONVIF (PTZ camera control)

- NumPy (numerical operations)

# 7. Workflow

- Capture frames from multiple RTSP cameras

- Detect persons using YOLO

- Track each person using DeepSORT

- Display bounding boxes for all tracked individuals

- User clicks on a person to select target

- System continuously tracks selected person

- PTZ camera adjusts position to follow the target

# 8. Current Limitations

- No persistent identity across cameras (basic or limited ReID)

- No database or historical tracking

- No floor map visualization

- No alerting system

- PTZ follows only one person at a time

# 9. Future Enhancements (Optional)

- Strong cross-camera ReID model (OSNet / FastReID)

- Multi-camera identity handoff

- Web-based dashboard

- Event logging and analytics

- Multi-target tracking with multiple PTZ cameras

# What we achieved

This is NOT just a demo — it is:

- Real-time AI system

- Multi-camera capable

- PTZ-integrated (this is rare and impressive)

- Interactive (user-in-the-loop tracking)

# References

- Demo Video

- Animated Presentation

- Demo PPT